#define LEFT_SENSOR   A0

#define MID_SENSOR   A1

#define RIGHT_SENSOR  A2

// Motor pins (same as yours)

#define M1A 3

#define M1B 5

#define M2A 6

#define M2B 9

// Parameters (tune these)

int threshold   = 500;  // sensor threshold (black/white)

int baseSpeed   = 120;  // forward speed (0-255)

int turnSpeed   = 80;   // gentle turn speed

int strongTurn  = 50;   // strong turn speed (or 0 for pivot-like)

void setup()

{

pinMode(M1A, OUTPUT);

pinMode(M1B, OUTPUT);

pinMode(M2A, OUTPUT);

pinMode(M2B, OUTPUT);

pinMode(LEFT_SENSOR, INPUT);

pinMode(MID_SENSOR, INPUT);

pinMode(RIGHT_SENSOR, INPUT);

// Keep motors in forward-only mode (B pins always 0)

analogWrite(M1B, 0);

analogWrite(M2B, 0);

}

// Helper: set motor speeds (forward only)

void drive(int leftPWM, int rightPWM)

{

leftPWM  = constrain(leftPWM,  0, 255);

rightPWM = constrain(rightPWM, 0, 255);

analogWrite(M1A, leftPWM);

analogWrite(M1B, 0);

analogWrite(M2A, rightPWM);

analogWrite(M2B, 0);

}

void loop()

{

int leftValue  = analogRead(LEFT_SENSOR);

int midValue   = analogRead(MID_SENSOR);

int rightValue = analogRead(RIGHT_SENSOR);

// Assume: value < threshold => BLACK line detected

bool L = (leftValue  < threshold);

bool M = (midValue   < threshold);

bool R = (rightValue < threshold);

// --- Decision logic ---

// 010: centered -> go straight

if (!L && M && !R)

{

drive(baseSpeed, baseSpeed);

}

// 110: slightly left -> turn right gently

else if (L && M && !R)

{

drive(turnSpeed, baseSpeed);

}

// 011: slightly right -> turn left gently

else if (!L && M && R)

{

drive(baseSpeed, turnSpeed);

}

// 100: far left -> strong right turn

else if (L && !M && !R)

{

drive(strongTurn, baseSpeed);

}

// 001: far right -> strong left turn

else if (!L && !M && R)

{

drive(baseSpeed, strongTurn);

}

// 111: all black (cross / junction) -> go straight (you can customize)

else if (L && M && R)

{

drive(baseSpeed, baseSpeed);

}

// 000: line lost -> stop

else

{

drive(0, 0);

}

}